## Cognome						Nome				n° matr.

# Section 1. TRUE/FALSE QUESTIONS

- The rotameter is one of the Differential Pressure Flowmeters

- true 	false 

- The membrane valve got this name just because has a ceramic membrane in contact with the flowing fluid

- true 	false 

- The Coriolis Flowmeter is a transducer of volumetric flowrate

- true 	false 

# Section 2. MULTIPLE CHOICE QUESTIONS

- When the percentage of flow through a valve equals the percentage of plug movement, a valve

a.   	Linear flow characteristic?

b.  	Equal percentage flow characteristic

c.  	Quick opening flow characteristic

d.  	Curved flow characteristic

- Which of the following parts of a globe valve serves the same pur pose as the disk in a butterfly valve?

a.  	Seat

b.  	Plug

c.  	Packing rings

d.  	Packing flange

- What is roughly a gauge pressure of 195 psi when converted in absolute psi ?

a.  	151

b.  	164

c.  	178

d.  	210

- In the processing industry, the most common control valve body style is

a.  	Butterfly

b.  	Three-Way

c.  	Globe

d.  	Bourdon

- Control valves that have a plug positioned by a stem, which slides through a packing gland, are often referred to as….?

a.  	rotary motion control valves

b.  	linear stem motion control valves

c.  	ball control valves

d.  	motorized control valves

- Butterfly valves are ….?

a.  	rotary motion control valves

b.  	linear stem motion control valves

c.  	ball control valves

d.  	motorized control valves

# Section 3: SENSORS AND INSTRUMENTATION FOR PROCESS MEASUREMENTS

## 3.1.	The thermocouple

- Describe in 3 rows its working principle

## 3.2.	Pressure

- Describe in 3 rows difference between absolute and gauge (gage) pressure

## 3.3.	The Level Sensors

- Describe in 3 rows the differences between their measuring principles

## 3.4.	The general properties of Sensors

- Describe in 3 rows definition and meaning of accuracy

# Section 4: VALVES

## 4.1 Valve

Explain the purpose and functioning of the following:

•	Globe control valves

•	Three-way valves

•	Butterfly control valves

(NB: A well organized and short table will be assessed more than a long and confused text!)

## 4.2.	Valve Technology

- Recognize the type of valve in the figure and write its application purpose

- Recognize the main characteristic parts of valve in the figure

(NB: It’s possible to indicate parts directly on the following figure)

## 4.3. Sizing Problem

A food industry need a new control valve for a process stream at the following conditions:

fluid: sunflower oil

density: f = 0.032 lb/in3

downstream pressure P2 = 29 psi

vapor pressure: Pv = 0.1 psi

FF = 0.956

nominal size of piping: DN = 60 mm

mass flow rate: m = 600 lb/min

upstream pressure: P1 = 52 psi

- With respect to the above range of variations, calculate the flow coefficient Cv for the valve working conditions that appear to be the most demanding

A steel-made, flange mounted globe valve Combraco 57 is available. It may adopt either a linear or equalpercentage or parabolic intrinsic characteristic. The rangeability  is r = 20. The valve manufacturer data are:

| DN mm | Kvn m3 (H2O) /h bar½ |
| --- | --- |
| 8 | 3.0 |
| 15 | 9.2 |
| 20 | 12.1 |
| 40 | 17.0 |
| 60 | 29.3 |
| 80 | 34.6 |

The conversion for Kv to the flow coefficient Cv is as follows:

Cv = 1.16 Kv [=] gpm (USA) psi-1/2

- What is the valve nominal size?

- Size the valve and choose that one with the most appropriate DN and intrinsic characteristic

- Check cavitation according to the IEC norm

The valve chosen above is to install in a plant circuit that has an overall pressure drop P0 = 3 atm = constant.

- What is the installed characteristic?

- Choose an optimal value for the valve authority V, which should be congruent with the previous choice of the intrinsic characteristic

- Draw a quantitative diagram that displays the flow rate ratio V(h)Vn just for the above chosen  valve authority

- For the same value of the valve authority V, calculate the nominal pressure drop across the valve Pn?

- Correspondingly, how much is the new value of the nominal flow rate Vn?