from the written test No.1 of June 12, 2012

- Section 3: SENSORS AND INSTRUMENTATION FOR PROCESS MEASUREMENTS

- 3.1.	Temperature measurement with thermocouple

A J-type thermocouple is used to measure the temperature of superheated steam.

The cold junction of the measuring circuit is at a temperature: Tcj = 305.16 K.

The voltmeter connected to the cold junction provides a measure of the electromotive force: V = 29.08 mV.

- determine the temperature Thj of the steam in °C with an approximation  ±1°C

### Data:

Tcj = 305.16 K = 32°C

## Solution

from the section with T (°C ) > 0 in the   “Reference table – Type J  TC”:

	E(Tcj) = 1.641 mV

Due to the Law of Successive or Intermediate Temperatures, the following eq. holds::

E(Thj) = V + E(Tcj) = (29.080 + 1.641) mV = 30.721 mV

By applying the inverse procedure from the “Reference table – Type J  TC”

	Thj = 559°C ± 1°C