## Cognome					Nome			n°matr.:

# Section 1: TRUE-FALSE QUIZZES

- The response time is the time interval which elapses before the step response reaches a value that differs no more than 6 % from its final steady-state value

true o	false o

- A 2nd order dynamical system is always equivalent to a series of two linear first order systems

true o	false o

- A command air pressure of 3-15 psi is typical of membrane actuators

- true o	false o

- A relay controller can operate without hysteresis

- true o	false o

# Section 2: MULTIPLE CHOICE QUIZZES

- The Laplace transform cannot be used to solve

a.  	second order differential equations

b.  	linear or linearised differential equations

c.  	nonlinear differential equations

d.  	higher-order linear differential equations

- Which of the following acronyms does not correspond to a fitting model for the response of a system with an unknown dynamics?

a.  	SOPDT overdamped

b.  	SO with Lead Time

c.  	SOPDT underdamped

d.  	FOPDT underdamped

# Section 3: REFERENCE DYNAMICAL MODELS

## 3.1	FOPDT model

- Show the characteristic parameters and write the transfer function.

- Is it a linear model?

- It can also be defined as a model of systems in series. Why?

- How would you classify its general dynamical behaviour in an open loop?

- Draw a quantitative plot of its response to a step variation of the input variable.

- Illustrate an example of a real system in process engineering whose transfer function is well approximated by a FOPDT model

# Section 4: PROCESS REGULATION AND CONTROL

## 4.1 Feedback control

Two liquid streams (stream  and stream ) with different contents of a compound P must be isothermally mixed in a vessel in order to obtain an unique stream whose concentration of P is equal to a given value (see figure below). A feedback control system must be designed for this process.

- Create a table in which all the components which are necessary for the control of this process are listed and briefly described;

- Draw the correspondent block diagram for the feedback control, writing on it the characteristic variables that are involved in this process.

NB: A short and well-structured answer is better than a long and confused one!

# Section 5: CONTROLLERS

- Write and explain the general law that regulate the behaviour of the controllers belonging to the PID family; furthermore, report their transfer function.

- What does direct action controller mean? And inverse action controller?

- Describe the different criteria of error minimisation which can be used for an optimal tuning.

- Describe the controller tuning method based on the analysis of the closed loop response.

NB: A short and well-structured answer is better than a long and confused one!

# Section 6: MATHEMATICAL MODELLING

In a mixer a liquid stream is heated by a flow rate VS of saturated steam at temperature TS, circulating in a heating jacket (see figure).

The following hypotheses are made:

- Ideal mixing in the vessel;

-  = const.

- TS = const.

- λ (latent heat) =const.

- Vin = const.

- the liquid height in the vessel is constant;

The variable that must be predicted by the model is the temperature Tout of the output  stream of the mixer. You must:

- write a steady state model

- write a dynamical model

- classify the obtained dynamical model

- list input, state, output variables and the parameters of the model

- discuss which input variables can be assumed as forcing functions and which are their possible functional forms for this physical problem

- write the model using the deviation variables

- find the correspondent form of this dynamical model in the Laplace domain

- write the transfer functions

- determine the order of this transfer function

As a further deeper examination of the process under study, consider the following case. The stream that leaves the mixer (whose temperature is Tout) goes through an insulated coil (see figure), which determines a dead time td . The temperature of the stream leaving the coil is Td.

- Which is the mathematical relationship between Td e Tout in the time domain?

- Which is the global transfer function for this new system (vessel + coil)?