## Student’s surname					Name

# Section 1: TRUE-FALSE QUIZZES

- The time constant p of a first order system is the time interval which elapses before the step response reaches the 50 % of its final steady-state value
	true o	false 

- The ISE tuning method requires the minimisation of the integral of the error norm
	true 	false o

- The Laplace transform can be applied only to linear dynamical systems
	true 	false 

- In the servo problem, the load is constant while the set point is variable in time
	true 	false o

- The zero of a rational transfer function is a value of s for which the denominator is equal to zero
	true 	false 

# Section 2: MULTIPLE CHOICE QUIZZES

- An industrial regulator can be:

NOTE: choose the wrong answer!

- 	a “direct acting” regulator

- 	an “inverse acting” regulator

- 	a relay regulator

- 	a FOPDT regulator

- Which of the following ones is not an actuator?

- 	hydraulic ram

- 	centrifugal pump

- 	relay controller

- 	heating or cooling element

- The transfer function of a PD controller is:

- 	Gc = Kc[1+ tds]

- 	Gc = Kc[1+1/(Ds)]

- 	Gc = Kc[1+Ds)]

- 	Gc = Kc/(Ds)

- The offset is:

- 	y͚͚͚∞ – ySP(t)

- 	ySP(t) – y∞

- 	y∞ – ym(t)

- 	always positive

- In the following simplified scheme of a car automatic control the disturbance is:

- 	steering wheel position

- 	throttle lever position

- 	wind drag

- 	car speed

Section 3: REFERENCE DYNAMICAL MODELS

## 3.1	Underdamped response of a 2nd order system

For this type of dynamic response:

- what is the mathematical model in the time domain?

- write the transfer function.

- define and explain the characteristic parameters.

- report poles of the 2nd order transfer function on a quality plot in a complex plane and illustrate their behaviour as  changes between 0 <  < 1

- plot the input step change response for a value of  on your choice.

- what is the overshoot? when does it occur?

- calculate the overshoot for  = 0.3

# Section 4: PROCESS REGULATION AND CONTROL

## 4.1. Feedback control

- Draw the Block Diagram for the most general case representing the feedback Control problem.

- Describe briefly the feedback control components one by one

- Describe briefly the meaning of the variables appearing one by one in the feedback control loop

# Section 5: CONTROLLERS

## 5.1 PID controller tuning

The following diagrams report the dynamical response (see the upper curve) to a step input (see the lower curve) for three processes whose dynamics is unknown. It is required to tune a PID control for each of them.

NOTE: only qualitative answers are required: numerical computation need not be performed!

- Which approximating dynamical model would you adopt for a process giving its step test response as in the following figure?

- List the parameters involved in this model

- Which tuning rules do you believe can be used in this case?

- Which approximating dynamical model would you adopt for a process giving its step test response as in the following figure?

- List the parameters involved in this model

- Which tuning rules do you believe can be used in this case?

- Which method has been used to generate this dynamical response (see output) in this third process?

- List the parameters involved in it

- Which tuning rules do you believe can be used in this case?

# Section 6: MATHEMATICAL MODELLING

## 6.1 Development of a mathematical model for a lumped-parameter dynamical system

2 non interacting tanks (see. Figure) are used for storage and distribution of a water solution of nutraceutical additives in a food factory.

The water solution is entering with  a temperature Ti(t) from the top of the tank N.1, with cross section S1, and is leaving with a temperature T2(t) tank N.2, with cross section S2.

The following hypotheses are assumed:

- Constant level in each tank

- The volumetric flow rate V is constant

- Perfect mixing in each tank

- each tank is thermally perfectly insulated from the surrounding environment and heat losses are negligible.

- constant physical properties  = const. e cp = const.

The variable that must be predicted by the model is the temperature T2(t).

You must:

- Write a steady state model

- Write a dynamical model

- Classify the obtained dynamical model

- list input, state, output variables and the parameters of the model

- discuss which input variables can be assumed as forcing functions and which are their possible functional forms for this physical problem

- write the model using the deviation variables

- find the correspondent form of this dynamical model in the Laplace domain

- write the transfer function

- Classify the transfer function

Let us assume a new hypothesis as a modification to the previous 3):

3’) The tank N.2 is not  perfectly mixed anymore: The tank temperature T*(t) is homogeneous inside it, but the outlet stream  presents a temperature T2(t) that is different and obtainable by the eq.:

T2t−TtT1t−Tt=exp(−γ)

where γ is a constant and known non-dimensional parameter.

- What is the new dynamical model for the tank N.2? Compare it to the previous one

- Is the model for the tank N.2 still linear?

- Does the order of the overall dynamical model (tank N.1 & tank N.2) change?