## Last name				Name				student ID No.:

# Section 4: STABILITY OF LINEAR DYNAMIC SYSTEMS

A dynamic process has the following linear transfer function

Gp=1ns3+s2+s+n2e−2ns

where n = PC No.

- Is this a minimum phase system?

## Part A: Root locus

- For the dead time, determine the Padè approximation according to the following table:

| order | n |
| --- | --- |
| 1 | 1  n  5 |
| 2 | 6  n  10 |
| 3 | 11  n  15 |
| 1 | 16  n  20 |
| 2 | 21  n  25 |
| 3 | 26  n  30 |
| 1 | 31  n  32 |

- Determine the new fully rational transfer function that you obtain for the process, Gp,approx, after replacing the dead time with the Padè approximation

Plot the root locus by means of Matlab and SisoTool resources, attach it and answer here the following questions:

- Determine poles, zeroes and No. of trajectories.

- Determine parts of root locus coincident with the real axis.

- Discuss existence of asymptotes and, if possible, calculate the gravity center and angles formed with the real axis.

- Discuss and, if possible, calculate starting and ending angles for poles and zeroes.

- Discuss and, if possible, calculate the breakaway points

- Discuss the BIBO stability of the system when Kc is changed

- Calculate the limiting value/values for Kc

- If the Padè approximation is 1st order, discuss the effect on BIBO stability due to the switch from a P controller to a PD controller. Otherwise, go to the next question

## Part B: Frequency response

For the original dynamic system (see Gp reported on top):

- Plot the Bode Diagrams by means of Matlab resources and attach them here

- Discuss the asymptotic behavior for low and high frequencies.

- Does a resonance frequency exist? How much is it?

- Does a crossover frequency exist? How much is it?

- Does a gain crossover frequency exist? How much is it?

- Check, on the base of the Bode stability criterion, if the above system is closed-loop stable and calculate the gain margin.

- Plot the Nyquist Diagram by means of Matlab resources and attach it here

- Check, on the base of the Nyquist stability criterion, if the above system is closed-loop stable

- Calculate the phase margin

- By changing Kc, calculate its limiting value and compare it with the previous value found under the Padè approximation

## Part C: Dynamic responses in the time domain

For the approximated dynamic system Gp,approx:

- If the Padè approximation is 2nd order, plot the open-loop and closed-loop system dynamic responses to a unit step change in set point by means of Matlab resources, attach them here and give your comments. Otherwise, go to the next question

## Part D: Dead time compensator

- Write its transfer function and give your comments