## Last name				Name				“matricola”:

# Section 4: STABILITY OF LINEAR DYNAMIC SYSTEMS

A dynamic process has the following linear transfer function

Gp=s+n+0.5n−0.52s2+n22

where n = PC No.

- Which order is this system?

- Is this a minimum phase system?

- How much is the gain?

- How many and how much are the time constants?

## Part 1: Open Loop Stability

- Plot the open loop step response in the time domain, attach it here and briefly comment it

- Is this a self-regulating system?

- Provide your best definition of self-regulating system

## Part A: Root locus

Plot the root locus by means of Matlab and SisoTool resources, attach it and answer here the following questions:

- Determine poles, zeroes and No. of trajectories.

- Do you use direct or inverse root locus rules? Why?

- Determine parts of root locus coincident with the real axis.

- Discuss existence of asymptotes and, if possible, calculate the gravity center and angles formed with the real axis.

- Discuss starting and ending angles for poles and zeroes, and calculate those for complex poles.

- Discuss and, if possible, calculate the breakaway points

- Discuss the BIBO stability of the system when Kc is changed

- Discuss the effects on BIBO stability due to the switch from a P controller to either a PD controller or a PI controller. Which one is the best? Why?

- Provide an example of another transfer function whose root locus never lies on the real axis

## Part B: Frequency response

Make use of Matlab and SisoTool resources and answer here the following questions for the above given linear transfer function:

- Plot the Bode Diagrams and attach them here

- Discuss the asymptotic behavior for low and high frequencies.

- Does a resonance frequency exist? How much is it?

- Does a crossover frequency exist? How much is it?

- Does a gain crossover frequency exist? How much is it?

- Check, on the base of the Bode stability criterion, if the above system is closed-loop stable and calculate the gain margin.

- Plot the Nyquist diagram and attach it here

- Check, on the base of the Nyquist stability criterion, if the above system is closed-loop stable

- Calculate the phase margin

## Part C: Dynamic responses in the time domain

For the above dynamic system Gp:

- For Kc=1, plot the closed-loop system dynamic response to a unit step change in set point by means of Matlab resources, attach here and give your comments.